# Beginning Farmer and 4-H Youth Agriculture Complex - Master Plan Study

## Draft Program Criteria Document

### Owner Review Draft

## 1. Purpose

This Program Criteria Document is intended to serve as the Owner-approved guide for the planning, design criteria, and facility requirements for the Beginning Farmer and 4-H Youth Agriculture Complex.

The document is intended to be referenced by the Owner, design team, consultants, and future contract documents as the basis for program development, conceptual planning, phasing, and preliminary opinions of probable cost.

This document is not a code analysis, construction document, or final design standard; however, all recommendations are intended to be feasible and buildable. Final design shall be coordinated with applicable codes, AHJ requirements, County standards, environmental constraints, utility availability, and funding limitations.

## 2. Project Goals

The project shall support the following primary goals:

- Provide a flexible agricultural education complex supporting 4-H Youth Development, beginning farmer/rancher training, livestock care, pasture management, and community education.

- Reuse and adapt the existing Red House as the primary operations, teaching, meeting, and outreach building.

- Reuse, repair, or selectively replace existing livestock structures where feasible, with priority given to function, safety, animal welfare, durability, and cost-effective phasing.

- Maintain and enhance the “Old Florida” rural character of the site, including reuse of existing materials where appropriate.

- Provide modular animal husbandry facilities that can respond to demand over time rather than locking the project into a fixed animal count.

- Establish Phase One improvements that can support limited, functional operation within available initial funding.

- Protect water quality, manage stormwater responsibly, and support appropriate use of agricultural lands within the Cow Pen Slough watershed.

- Integrate best practices from the Guide for the Care and Use of Agricultural Animals in Research and Teaching, Fourth Edition, as the animal-care performance standard for husbandry, housing, health, biosecurity, handling, and environmental conditions, and any other resources as appropriate.

## 3. General Program Criteria

### 3.1 Planning Approach

The facility shall be planned as a working agricultural campus with clear separation between public, staff, instructional, animal-care, service, and pasture functions.

The site plan shall support:

- Safe student, staff, visitor, trailer, and service vehicle circulation.

- Controlled access to animal-care areas.

- Modular expansion of barns, pens, pastures, and support areas.

- Efficient daily care, cleaning, feeding, washdown, veterinary access, and emergency response. Facilitate ease of maintenance access and operations.

- Reasonable and required supervision of youth participants and public visitors.

- Event hosting for Farm to Table experiences, arena shows, or other community and volunteer events.

- Clear phasing that distinguishes minimum functional improvements from future expansion.

### 3.2 Animal Care Standard

Animal husbandry spaces shall be planned to support the following Ag Guide principles:

- Animals shall have appropriate access to feed, clean water, shelter, ventilation, shade, sanitary housing, and species-appropriate social conditions.

- Space shall be sufficient for normal postural adjustments, including standing, lying, resting, eating, drinking, and elimination.

- Facilities shall support daily observation and timely identification of illness, injury, stress, or unsafe conditions.

- Facilities shall support appropriate segregation or temporary holding of sick, injured, newly arrived, or incompatible animals.

- Feed, bedding, water, waste, and chemicals shall be managed to reduce pests, contamination, odors, disease transmission, and water-quality impacts.

- Handling and restraint facilities shall be species-appropriate and arranged to reduce stress, injury, and unsafe interactions between animals and handlers.

- Biosecurity practices shall be supported through circulation, separation, cleanable surfaces, pest control, and operational procedures.

### 3.3 Modularity

Animal housing shall be designed for adaptable use. Pens, panels, gates, alleys, water systems, and support spaces shall allow reconfiguration as program demand changes.

The Owner’s intent is to accommodate individual applicant needs on a first-come, first-served basis where practical. Therefore, the program should avoid over-specialized fixed layouts unless required for safety, welfare, or operational efficiency.

3.4 Dimensional Basis of Planning

Animal housing dimensions shall use The Farm’s established local planning modules as the initial basis of design: 12'-0" x 24'-0" cattle pens and 6'-0" x 10'-0" small animal pens, unless revised by the Owner during program confirmation.

These modules are intended to serve as planning standards for conceptual design, phasing, and cost estimating. They shall be evaluated against the Ag Guide’s species-specific floor-area, handling, feeding, watering, ventilation, sanitation, and animal welfare criteria based on final animal type, size, group size, and duration of housing.

Where the Ag Guide provides performance-based space allowances rather than fixed pen dimensions, the design team shall document the planning assumptions used to confirm adequacy of each pen module, including:

- Animal type and anticipated body size or weight range.

- Number of animals per pen.

- Whether animals are housed individually or in groups.

- Bedding, flooring, and drainage system.

- Feed and water access.

- Duration of housing, including seasonal or year-round use.

- Required separation, isolation, temporary holding, or quarantine functions.

- Operational requirements for cleaning, observation, handling, and safe animal movement.

The intent is to use the Ag Guide as the animal-care performance standard, while using The Farm’s proven local pen modules as the Owner-reviewed dimensional precedent for this project.

### 3.5 Durability

Animal-care facilities shall be designed for heavy agricultural use, including water exposure, impact, corrosion, manure, bedding, rodents, insects, and daily cleaning.

Preferred durability criteria include:

- Galvanized or corrosion-resistant animal panels, gates, fasteners, and hardware.

- Heavy-duty outdoor-rated fans and fixtures.

- Cast iron or similarly durable automatic waterers where appropriate.

- Exposed piping protected from damage, with metal piping and durable couplings in animal areas.

- Secured pendant fixtures and utilities protected from animal contact and wind movement.

- Deep roof overhangs, gutters, or other perimeter drainage strategies to reduce bedding erosion and localized flooding.

- Cleanable and rodent-resistant feed, tack, and storage areas.

### 3.6 Security and Access Control

The facility shall support secure after-hours operation and appropriate protocol for supervision of youth participants.

Criteria shall include:

- Controlled vehicular access.

- Trailer staging and service access.

- Security cameras at barn aisles, gates, and key public/service areas and access points.

- Coordination with Sarasota County General Services for security expectations.

- Clear distinction between public-facing areas and restricted animal-care/service areas.

### 3.7 Sustainability and Water Quality

The project shall incorporate County sustainability requirements, Florida-Friendly LandscapingTM, and UF/IFAS agricultural best practices where ever possible.

Criteria shall include:

- Follow green building and renovation principles wherever possible, in line with County policy and standards.

- Support the use of the facility and its landscape as demonstration areas for educating the public on best practices.

- Water-quality protection and improvement as a primary site-planning concern.

- Stormwater strategies appropriate for animal-care areas, washdown, roof runoff, drives, and pasture drainage.

- Integrated Pest Management.

- Low-maintenance native or adapted landscaping, following and demonstrating Florida-Friendly LandscapingTM principles.

- Water use and energy reduction strategies maximized and put forth as standards expected in design, including ENERGY STAR and WaterSense products.

- Plan the campus to be solar-ready, including structural loading, roof orientation, and electrical infrastructure and equipment, and utility connection. Include addition of solar as an option depending on funding.

- Practical, durable, low-energy passive design strategies, including shade, natural ventilation, roof overhangs, and building orientation.

- Waste reduction strategies integrated in construction and operations, including onsite reuse, high recycled content products, onsite composting facilities, and more.

## 4. Building and Space Program

# A. Red House: Operations, Education, and Outreach Building

## A.1 General Role

The Red House shall be reused and converted to provide the primary public-facing and administrative functions for the complex.

The building shall support:

- 4-H Youth Development programs.

- Beginning farmer/rancher education.

- Community training and outreach.

- Staff and volunteer operations.

- Meetings and small conferences.

- Demonstration and cottage-industry kitchen activities.

The design shall maintain the existing open-plan character where feasible and preserve or enhance the Old Florida character of the structure.

## A.2 Lobby / Reception

### Function

Primary arrival and orientation space for visitors, students, families, staff, and community users.

### Criteria

- Provide a clear public entry sequence.

- Include space for check-in, display, notices, wayfinding, and program information.

- Provide direct access to public meeting/training spaces and restrooms.

- Maintain visibility and supervision from staff areas where feasible.

- Architectural character should reinforce the rural Old Florida identity of the campus.

## A.3 Staff Offices

### Function

Office space for approximately five staff members.

### Criteria

- Provide private workstations with closing doors as appropriate to the existing structure. Semi-private is acceptable for collaborative roles. Provide flexible workstations for additional staff if space allows.

- Locate near lobby/reception where practical for visitor control and daily operations.

- Provide access to files, printer/copier, supplies, and secure storage.

- Include visual or operational connection to public entry and meeting spaces where feasible.

- Accommodate County technology and network infrastructure standards.

- Provide space for a cot or sleeper couch to enable staff to stay overnight (rarely) if needed for animal care reasons. Consider including space for an onsite caretaker to live all or part of the year, possibly through a pad with electrical, water, and sewer connections to start and eventually a small residence.

## A.4 Conference / Meeting Room

### Function

Meeting and conference space for approximately 20 to 25 people.

### Criteria

- Support staff meetings, advisory meetings, small-group training, volunteer coordination, and partner meetings.

- Provide flexible table and chair arrangements and associated furniture storage space.

- Include display/AV capability.

- Locate near lobby and restrooms.

- Acoustically separate from louder teaching, kitchen, or service areas where feasible.

## A.5 Instructional Classroom / Training Workshop

### Function

Large teaching and workshop space for approximately 50 people.

### Criteria

- Provide the largest practical well-proportioned room within the existing building constraints.

- Support classroom seating, workshop tables, demonstrations, presentations, and community training.

- Provide flexible furniture layout and associated furniture storage space.

- Include durable finishes suitable for agricultural education use.

- Provide AV/display capability, storage for teaching materials, and access to restrooms.

- Structural spans and existing floor-level changes shall be evaluated early because they may affect room size, proportions, accessibility, and cost.

## A.6 Demonstration and Commercial / Cottage-Industry Kitchen

### Function

Kitchen space to support demonstration, teaching, and cottage-industry food preparation standards as determined by the Owner and applicable reviewing agencies.

### Criteria

- Maintain the preferred open-plan concept where feasible, able to be closed off with a large door opening as needed.

- Provide one-direction workflow for dishwashing and food handling where practical.

- Include adequate handwashing, warewashing, food-prep, storage, refrigeration, and clean/dirty separation as required by the final program and Health Department review.

- Coordinate final equipment requirements with the intended commercial kitchen standard.

- Confirm whether the kitchen is intended for demonstration only, limited processing, public classes, certified production, or other use requiring agency review.

- Provide durable, cleanable finishes and pest-resistant detailing.

- Coordinate grease waste, plumbing, ventilation, electrical load, and fire protection requirements during design.

## A.7 Breakroom

### Function

Staff and volunteer support space.

### Criteria

- Low-priority; provide as space allows.

- Provide seating, kitchenette or break counter, refrigerator, microwave, and storage.

- Locate convenient to staff offices but separate from public kitchen operations.

- May share infrastructure with other support spaces if practical.

## A.8 Restrooms

### Function

Code-compliant restroom facilities serving staff, public visitors, training events, and site users.

### Criteria

- Provide accessible restrooms as required by code and occupancy.

- Locate for convenient access from lobby, classroom, meeting room, and outdoor program areas.

- Evaluate whether restrooms can also support exterior users without compromising building security.

- Coordinate fixture counts with final occupant loads and AHJ interpretation.

- Try to maintain a shower for staff use.

## A.9 Storage / Support

### Function

Support storage for teaching materials, office supplies, cleaning supplies, tables, chairs, and event equipment.

### Criteria

- Provide distributed storage rather than relying only on barn or maintenance areas.

- Storage is primarily comprised of large plastic stackable bins.

- Include janitor closet with mop sink where feasible.

- Separate food/kitchen storage from cleaning and chemical storage.

- Provide secure storage for equipment that should not be accessible to youth participants or visitors.

## A.10 Accessibility and Existing Building Constraints

### Criteria

- Address multiple floor levels with accessible routes as required.

- Evaluate feasibility of ramps, lifts, regrading, or limited-use area assignments.

- Confirm existing structural capacity and floor plan modifications required to create large teaching spaces.

- Conduct or require pest inspection during the first design phase after the study.

- Confirm septic tank, drain field, well, and utility locations before finalizing reuse strategy.

# B. Primary Barn: Livestock Housing / Adaptive Reuse Structure

## B.1 General Role

The existing primary barn shall be evaluated for reuse as an animal housing and support structure. If the structure cannot be salvaged, the Owner desires salvage and reuse of rough-sawn cypress siding and pecky cypress casing/trim where practical.

Potential uses may include cattle support, hog pens, stabling, equipment storage, event space, or flexible animal housing depending on structural condition, environmental constraints, and program phasing.

## B.2 General Barn Criteria

- Confirm structural feasibility, clearances, roof condition, lateral stability, termite/pest damage, and suitability for animal occupancy.

- Preserve character-defining agricultural materials where feasible.

- Maintain large openings and vehicular clearance where useful for bedding, feed delivery, equipment, and cleaning.

- Provide ventilation, shade, and weather protection.

- Provide durable, washable, rodent-resistant, and corrosion-resistant materials in animal areas.

- Coordinate electrical grounding, bonding, lightning protection, and safe separation of animals from electrical equipment.

- Provide security camera coverage at aisle ends and major entries where feasible.

- Include area for storage of trailer, kayaks and other equipment storage needs.

- Consider area for cattle chute and scale.

## B.3 Flexible Pen Areas

### Function

Modular animal housing capable of supporting cattle, swine, goats/sheep, or temporary holding, subject to final Owner direction.

### Criteria

- Use modular galvanized panels and gates where feasible.

- Coordinate gate swing direction to support multiple cleaning, feeding, sorting, and emergency scenarios.

- Provide adequate aisle width for tractor or equipment access for bedding replacement.

- Provide sand, bedding, or other species-appropriate floor/bedding systems over a drainage base where appropriate.

- Provide durable automatic waterers and protected piping.

- Avoid exposed PVC fittings in animal-contact areas.

- Provide fans and ventilation suited to outdoor/agricultural conditions.

- Provide roof overhangs, gutters, or drainage improvements to reduce erosion and bedding washout.

- Provide temporary holding areas for cleaning, separation, illness, or program flexibility.

## B.4 Hog Pen Area

### Function

Housing or shelter for swine if the existing primary barn is suitable.

### Criteria

- Design pens for durable containment, cleanability, shade, ventilation, water access, and safe handling.

- Provide a cool, shaded, well-ventilated environment with options for heat-stress mitigation.

- Provide secure gates and panels suitable for swine behavior and strength.

- Provide bedding or flooring appropriate to program use and manure management.

- Locate feed and water to reduce competition and maintain clean conditions.

- Avoid reuse of existing deteriorated west-side hog pens unless they can be fully reconstructed to current best-practice standards.

- Provide isolation or temporary holding capability.

## B.5 Horse Stabling / Large Animal Use

### Function

Stabling or temporary holding for horses or other large animals if included in the final program.

### Criteria

- Provide safe stall or pen dimensions based on final animal type and size.

- Avoid sharp projections, unsafe gaps, unstable doors, or protruding hardware.

- Provide adequate ventilation, shade, dry footing, and clean water.

- Provide separation from incompatible species or animals as needed.

- Provide secure tack/feed storage nearby.

- Provide safe lead-in/lead-out circulation connected to paddocks, washdown, and trailer access.

## B.6 Feed / Tack / Locker Storage

### Function

Storage for feed, tack, tools, animal-care supplies, and individual project lockers.

### Criteria

- Provide dedicated storage associated with animal housing areas.

- Lockers may be assigned by pen or project, based on Owner operating model.

- Design for rodent resistance and cleanability.

- Provide separate, secure feed storage areas distinct from individual lockers, with feed stored off the floor and away from walls on racks or pallets.

- Bulk or loose feed (i.e., feed not in sealed manufacturer packaging) shall be stored in sealed, rodent-resistant containers.

- Separate chemicals, medications, pesticides, and cleaning materials from feed.

- Provide adequate ventilation and moisture protection.

# C. Unfinished New Barn

## C.1 General Role

The unfinished new barn appears to be structurally complete and in generally good condition, but currently lacks power and water. Its final role shall be determined after survey, wetland, and environmental review.

Potential uses include cattle housing, flexible livestock shelter, equipment support, shade structure, or future expansion.

## C.2 Criteria

- Confirm whether the structure is within or adjacent to wetland limits.

- Confirm whether utilities can be provided without unacceptable environmental impact or cost.

- Evaluate structural condition, code status, and feasibility of occupancy or agricultural use.

- Provide water, power, lighting, ventilation, and drainage if used for animal housing.

- Design for modular pens and future expansion where feasible.

# D. Cattle Barn / Large Animal Housing

## D.1 General Role

Provide year-round or seasonal cattle project housing, beginning rancher support, and flexible large-animal care.

The program shall recognize that cattle projects may be year-round and demand-driven rather than seasonal only.

## D.2 Cattle Pens

### Baseline Criteria

- Use 12’-0” x 24’-0” cattle pens as the preferred planning module unless modified by final Owner direction.

- Provide sand bed over drainage base or equivalent durable, cleanable bedding/floor system.

- Provide galvanized panels and gates.

- Coordinate gate swing for feeding, cleaning, sorting, turnout, emergency access, and flexible pen combining.

- Provide cast iron or similarly durable automatic waterers.

- Use protected metal piping and durable couplings in exposed animal areas.

- Avoid exposed PVC fittings in animal-contact areas.

- Provide heavy-duty outdoor-rated fans.

- Provide roof overhangs, gutters, or drainage features to reduce bedding erosion and perimeter flooding.

- Maintain center aisle width adequate for tractor access and bedding replacement.

## D.3 Cattle Handling / Squeeze Chute / Scale

### Function

Safe handling, weighing, veterinary observation, and teaching.

### Criteria

- Provide a permanent or semi-permanent squeeze chute and scale arrangement.

- Keep chute and scale integrated to avoid damage to loose scales and reduce animal handling inefficiency.

- Locate in a controlled area connected to cattle pens, turnout, pasture, trailer access, and rough stock corral.

- Provide safe handler escape routes and protected working zones.

- Design alleys and gates to minimize animal stress and reduce sharp turns or visual distractions.

- Confirm final handling layout with Owner.

## D.4 Rough Stock Corral

### Function

Support incorporation of range cattle into the pasture grazing program.

### Criteria

- Provide or reserve space for a rough stock cattle corral if confirmed by Owner.

- Locate with direct relationship to pasture, trailer access, and handling chute.

- Provide robust fencing, gates, sorting alleys, and holding pens appropriate for less-acclimated cattle.

- Provide shade and water where animals may be held.

- Coordinate with electrical fencing assumptions and AHJ/site requirements.

## D.5 Washdown Area

### Function

Washing and grooming of cattle and other livestock.

### Criteria

- Provide limited wash space to support teaching, coordination, and shared use.

- Locate between or near barns where practical.

- Provide shade, especially for late afternoon use.

- Provide durable non-slip surface, drainage, hose bibbs, and hose management.

- Coordinate wash water collection, stormwater, manure management, and water-quality requirements.

- Provide lighting for late-day use if needed.

- Design to avoid uncontrolled discharge to wetlands, pastures, or drainageways.

# E. Small Animal Barn

## E.1 General Role

Provide flexible housing for small livestock, including sheep, goats, poultry, and other 4-H project animals as determined by Owner demand.

## E.2 Small Animal Pens

### Baseline Criteria

- Use 6’-0” x 10’-0” small animal pens as the preferred planning module unless modified by final Owner direction.

- Provide modular galvanized panels and secure gates.

- Provide cast iron or similarly durable waterers where appropriate.

- Provide protected piping and fittings.

- Provide offset heater configuration in pens where required for young or cold-sensitive animals.

- Secure pendant fixtures and utilities against wind and animal contact.

- Provide barn curtains with anti-billowing cables and straps where open-sided weather protection is required.

- Provide temporary holding or corner pens for cleaning, tool storage, or separation.

## E.3 Sheep / Goat Housing

### Criteria

- Provide dry, shaded, well-ventilated shelter.

- Provide clean water and feed access with reduced competition.

- Provide modular pens to allow grouping, separation, kidding/lambing if needed, and temporary isolation.

- Provide durable fencing and gates appropriate to goats’ climbing and escape behavior.

- Provide environmental enrichment opportunities for goats, such as durable climbing elements, only where they do not create safety or cleaning problems.

- Provide cleanable bedding/flooring and manure management.

- Separate incompatible animals and new arrivals as required by operating procedures.

## E.4 Poultry / Chicken Coop Area

### Criteria

- Provide secure predator-resistant coops or poultry housing.

- Provide ventilation, shade, clean water, feed storage, roosting/perching as appropriate, nesting as applicable, and cleanable litter management.

- Provide secure enclosure to prevent escape and reduce contact with wild birds.

- Support daily observation, egg collection where applicable, cleaning, and pest control.

- Provide separation from large animal areas where practical to reduce disease and stress.

- Coordinate final poultry space, density, nesting, and equipment requirements with final species, flock size, and production/teaching goals.

## E.5 Rabbit / Other Small Project Animal Area, if included

### Criteria

- Provide shaded, well-ventilated, predator-resistant housing.

- Provide secure cages or pens suited to the species.

- Provide clean water, feed storage, and cleanable waste management.

- Locate to reduce heat stress and direct sun exposure.

- Confirm final requirements with Owner before including in Phase One.

# F. Pastures and Livestock Habitats

## F.1 General Role

Provide pasture and habitat areas for livestock grazing, turnout, teaching, rotational grazing, and beginning rancher training.

## F.2 Pasture Planning Criteria

- Use rotational grazing to improve pasture capacity and recovery over time.

- Plan fencing and gates to allow flexible pasture rotation.

- Provide shade, water, and access routes appropriate to the species and stocking intensity.

- Pastures in wetland or seasonally wet areas shall be used only when ground conditions are appropriate.

- Grazing activity shall be timed to avoid damage to wet areas, reduce water-quality impacts, and support pasture recovery.

- Provide central water trough enclosures where feasible to reduce utility runs, simplify herd logistics, and acclimate cattle to smaller enclosures.

- Confirm pasture plan with Owner.

## F.3 Fencing

### Criteria

- Confirm whether electrical fencing is allowable on this site.

- Provide durable perimeter fencing and internal cross-fencing suited to livestock type and public safety, consistent with USDA NRCS standards for high-tensile fencing.

- Design gates for trailer access, tractor access, animal movement, emergency access, and pasture rotation.

- Provide safe separation between public areas and livestock areas.

- Coordinate fence alignments with wetlands, easements, drainage, trees, and future expansion.

## F.4 Watering Stations

### Criteria

- Provide clean water to livestock under all environmental conditions.

- Design troughs and waterers for durability, cleanability, and animal access.

- Protect piping and fittings from animal damage.

- Locate to support pasture rotation and avoid excessive mud, erosion, or congregation in sensitive areas.

- Consider centralized trough enclosures between pastures where feasible.

## F.5 Existing Pond

### Criteria

- Investigate the existing pond to determine purpose, hydrology, environmental status, safety concerns, and potential use.

- Do not assume the pond is suitable for livestock watering, stormwater, irrigation, or educational use until evaluated.

- Provide safety separation or fencing if required.

# G. Arena / Turnout / Demonstration Area

## G.1 General Role

Provide flexible outdoor or covered space for animal demonstration, training, turnout, youth instruction, and community events.

## G.2 Criteria

- Maintain a clear barn-to-turnout-to-access-gate circulation sequence.

- Support animal movement without crossing primary public pedestrian routes where feasible.

- Provide flexible open area suitable for multiple uses.

- Consider shade, lighting, seating, water, and power if included in future phases.

- Coordinate surfacing, drainage, fencing, and gates for animal safety and event flexibility.

- Design for future expansion if not included in Phase One.

# H. Horticulture / Shade Structure Area

## H.1 General Role

Provide a small, low-cost horticultural teaching area if supported by funding and final Owner priorities.

## H.2 Criteria

- Use simple shade structure or similar low-cost covered area.

- Support small-scale growing, demonstration, and teaching.

- Reserve room for future farming program expansion where feasible. Do not allow horticulture scope to compromise the primary animal husbandry and Red House reuse goals in Phase One.

# I. Kennel / Ancillary Existing Structures

## I.1 General Role

Existing kennel and ancillary structures shall be evaluated for potential reuse, removal, or future support functions.

## I.2 Criteria

- Confirm structural condition, code status, utilities, sanitation, and program value.

- Reuse only if the structure supports the program without creating maintenance, safety, pest, or animal welfare concerns.

- Potential uses may include storage, quarantine support, maintenance, or removal for site clarity.

# J. Site Infrastructure and Support Areas

## J.1 Vehicular Entry / Trailer Access

### Criteria

- Provide controlled vehicular access.

- Provide adequate turning and staging for trailers.

- Consider paving for full trailer staging length on both sides of gate if feasible.

- Provide convenient service access to barns, dumpster, feed delivery, maintenance, and emergency vehicles.

- Maintain or improve existing access drive where feasible.

## J.2 Parking

### Criteria

- Provide parking for staff, visitors, volunteers, meetings, and training events.

- Confirm existing parking status and zoning/code requirements.

- Locate parking to avoid conflict with trailer circulation and animal movement.

- Provide accessible parking and accessible route to the Red House.

- Maintain permeability wherever practical, recognizing high-use, high-weight vehicles may need more stable infrastructure.

## J.3 Dumpster / Waste / Manure Management

### Criteria

- Provide service access to dumpster and waste areas.

- Locate waste storage away from public entry and sensitive environmental areas.

- Provide manure/bedding handling strategy appropriate to animal operations, water quality, pest control, and County requirements.

- Maintain clean site practices to reduce rodents, insects, odors, and nuisance conditions.

- Set aside space for composting area.

## J.4 Utilities

### Criteria

- Confirm potable water, sanitary sewer or septic, reclaimed water, FPL, fiber/telecom, and other utility availability.

- Confirm well condition and capacity.

- Confirm septic tank and drain field location, capacity, and suitability for proposed occupancy and kitchen use.

- Provide power and water to animal-care structures as required for Phase One operation; design infrastructure with future phases in mind.

- Coordinate exterior lighting, security, cameras, and network infrastructure with County standards.

## J.5 Stormwater and Drainage

### Criteria

- Address roof runoff, washdown, drives, parking, barns, pastures, and animal waste areas.

- Avoid uncontrolled runoff from animal-care areas to wetlands, drainageways, or water bodies.

- Provide erosion control at barn perimeters.

- Coordinate with SWFWMD and local requirements as part of design development.

- Apply best practices and even beyond best practice innovative approaches to ensure water quality is protected and where possible improved.

## J.6 Life Safety / Fire Protection / Lightning Protection

### Criteria

- Perform preliminary fire code and life safety review during the study.

- Confirm code occupancy, occupant loads, required exits, accessibility, fire protection, fire alarm, and emergency access during design.

- Provide grounding/bonding for metal animal husbandry structures as required by code, AHJ, and engineering design.

- Provide lightning protection where required or recommended based on risk, existing conditions, and Owner direction.

## 5. Phase One Criteria

Phase One shall identify the minimum functional scope required to support limited site operation, building occupancy, and animal housing/care.

Phase One should prioritize:

- Safe and code-compliant reuse of the Red House for essential staff, training, meeting, restroom, and kitchen functions.

- Minimum livestock housing/care improvements required for initial 4-H and beginning rancher operations.

- Water, power, fencing, access, drainage, and security improvements necessary for safe operation.

- Animal welfare, water quality, and daily care requirements.

- Reuse of viable existing structures where feasible.

- Deferral of nonessential expansion, specialized program areas, and high-cost improvements to later phases.

## 6. Future Phase Criteria

Future phases may include:

- Additional modular barns or pen capacity.

- Expanded cattle, swine, poultry, sheep/goat, or horse facilities.

- Covered arena or enhanced demonstration, outdoor teaching area.

- Expanded horticulture/farming program.

- Additional meeting/classroom capacity.

- Expanded trailer parking and event infrastructure.

- Additional pasture improvements and rotational grazing infrastructure.

- Permanent maintenance, equipment, feed, and manure-management facilities.

## 7. Items Requiring Confirmation

The following items require confirmation:

- Phase One priorities beyond minimum functional scope.(I.E. Species accommodation)

- Whether electrical fencing is allowable on this site.

- Kitchen regulatory standards that the design will be based on.

- Health Department requirements for kitchen, septic, water, and food-related activities.

- Wetland limits and whether the unfinished new barn and Red House is affected.

- Septic tank and drain field location/capacity.

- Well location, condition, capacity, and water quality.

- Final parking, accessibility, and life-safety requirements.

- Whether the existing primary barn and unfinished new barn are structurally and code-feasible for reuse.

- Whether existing pond has any intended program, stormwater, irrigation, livestock, or educational function.

- Required County security, network, access control, and camera standards.

- Required level of lightning protection, grounding, and bonding for animal structures.